Raspberry Pi Screen Matching Process

1. Clone the Raspberry Pi Linux source code

1.1 Install / update git

Command：sudo apt update

Command：sudo apt install git

1.2 Clone the Linux kernel

Command：git clone --depth=1 https://github.com/raspberrypi/linux.git

2. File copy / replacement import

2.1 Device tree source file: vc4-kms-dsi-ts8550b.dts

Path：/home/"username"/linux/arch/arm/boot/dts/overlays/

2.2 Driver source code: panel-sitronix-st7701.c

Path：/home/"username"/linux/drivers/gpu/drm/panel/

3. Modify kernel build configuration (Makefile, Kconfig)

3.1 Makefile

Path：drivers/gpu/drm/panel/Makefile

3.2 Kconfig

Path：drivers/gpu/drm/panel/Kconfig

Note: In this example, an existing internal driver is modified, so this step may be skipped. If you add a new panel driver, you must update these files.

4. Enable the driver using menuconfig

4.1 Install required dependencies

Command：sudo apt install libncurses-dev libncursesw5-dev flex bison pkg-config

4.2 Run menuconfig

Command：make menuconfig

4.3 Navigate and enable

Device Drivers  --->
  Graphics support  --->
    <M> Direct Rendering Manager (X.Org support)  --->
      Display Panels  --->
        <M> Sitronix ST7701S DSI panel

Note: This step is enabled by default and can be omitted. You can execute the check.

5. Compile the module

5.1 Compile the initialization C file to generate .ko

Command：make -j6 Image.gz modules dtbs

5.2 Expected output

CC [M]  drivers/gpu/drm/panel/panel-sitronix-st7701.o

LD [M]  drivers/gpu/drm/panel/panel-sitronix-st7701.ko

5.3 Install the module into the system

Command：make modules

Command：sudo make modules_install

Command：sudo depmod -a

6. Compile the device tree to generate .dtbo

6.1 Enter the overlay source directory

Command：cd /home/"username"/linux/arch/arm/boot/dts/overlays/

6.2 Compile .dts to .dtbo

Command：sudo dtc -@ -I dts -O dtb -o /boot/firmware/overlays/vc4-kms-dsi-rg-arc-panel.dtbo vc4-kms-dsi-rg-arc-panel.dts

7. Configure /boot/config.txt to load the overlay

7.1 Open config.txt

Command：sudo nano /boot/firmware/config.txt

7.2 Add the following lines

dtoverlay=vc4-kms-v3d-pi4

dtoverlay=vc4-kms-dsi-rg-arc-panel